### SUBCHAPTER D. NEIGHBORHOOD PLAN COMBINING DISTRICTS.

#### ARTICLE 1. GENERAL PROVISIONS.

§ 25-2-1401 APPLICABILITY OF SUBCHAPTER.

This subchapter applies to property that is:

(1)	located in a neighborhood plan (NP) combining district; and

(2)	used or developed as a special use described in Section 25-2-1403 (Special Uses).

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1402 SUPERSEDES OTHER REGULATIONS.

For property used or developed as a special use described in Section 25-2-1403, (Special Uses):

(1)	the regulations prescribed by this subchapter supersede the other provisions of Title 25 (Land Development) to the extent of conflict; and

(2)	the following regulations do not apply:

(a)	Section 25-2-514 (Open Space Standards);

(b)	Section 25-2-775 (Townhouses); and

(c)	Section 25-2-776 (Condominium Residential Use).

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1403 SPECIAL USES.

(A)	This section describes special uses for a NP combining district.

(B)	In this subchapter:

(1)	NEIGHBORHOOD URBAN CENTER special use is the use of a designated area within a neighborhood for commercial uses and townhouse, condominium, and multifamily residential uses.

(2)	CORNER STORE special use is the use of a site to provide goods or services to local residents.

(3)	COTTAGE special use is the use of a site of limited size for a single-family residential dwellings on lots at least 2,500 square feet in size.

(4)	NEIGHBORHOOD MIXED USE BUILDING special use is the use of a building for both commercial and residential uses.

(5)	RESIDENTIAL INFILL special use is the use of a designated area within a neighborhood for predominately residential uses and limited commercial uses.

(6)	SECONDARY APARTMENT special use is the use of a developed single-family residential lot for a second dwelling.

(7)	URBAN HOME special use is the use of a site for a single-family residential dwelling on a lot at least 3,500 square feet in size.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1404 OPEN SPACE.

(A)	In this subchapter:

(1)	COMMUNITY OPEN SPACE means open space that is available for use by the public, and includes a plaza, square, park, playground, greenbelt or similar area.

(2)	PRIVATE OPEN SPACE means open space that is associated with a dwelling and intended to be used exclusively by the dwelling's residents.

(B)	This subsection prescribes community open space requirements.

(1)	Community open space must be located within the boundaries of the special use for which it is required.

(2)	The portion of a drainage or water quality facility that is usable by the public for recreational purposes, as determined by the director of the Neighborhood Planning and Zoning Department, may be designated as community open space.

(3)	The community open space requirements of this subchapter are in addition to the parkland dedication requirement of Chapter 25-4, Article 3, Division 5 (Parkland Dedication), if applicable.

(C)	This subsection prescribes private open space requirements.

(1)	An area of private open space on ground level must be at least 10 feet across in each direction.

(2)	An area of private open space above ground level must be at least five feet across in each direction.

Source: Ord. 000406-81; Ord. 010329-18; Ord. 031211-11.

§ 25-2-1405 APPLICATION CONSOLIDATION; SPECIAL PROCEDURES.

(A)	The City may consolidate an application to designate a neighborhood as a NP combining district with an application to modify the zoning base district of an individual parcel within the neighborhood.

(B)	This subsection applies if the City elects to consolidate applications under Subsection (A).

(1)	The consolidated applications are considered one application for the purposes of notice and hearing.

(2)	For a zoning protest relating to a designation of a neighborhood as a NP combining district, the required percentage of land area is based on the boundaries and area of the entire proposed district.

(3)	For a zoning protest relating to a modification of the zoning base district of an individual parcel, the required percentage of land area is based on the boundaries and area of the individual parcel.

(4)	A separate council vote is required for:

(a)	a protest under Paragraph (2); and

(b)	each protest under Paragraph (3).

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1406 ORDINANCE REQUIREMENTS.

An ordinance zoning or rezoning property as a NP combining district:

(1)	must prescribe the special uses described in Section 25-2-1403 (Special Uses) that are permitted in the district;

(2)	must describe the location of each residential infill special use, neighborhood urban center special use, or neighborhood mixed use building special use, if any;

(3)	may restrict the time of day during which a business in a neighborhood mixed use building special use may be open to the public;

(4)	may restrict a corner store special use, cottage special use, secondary apartment special use, or urban home special use, if any, to a designated portion of the district;

(5)	for a single-family residential use on an existing legal lot platted on or before August 15, 2024 or a secondary apartment special use on an existing legal lot:

(a)	may reduce the required minimum lot area to 2,500 square feet;

(b)	may reduce the required minimum lot width to 25 feet;

(c)	for a lot with an area of 4,000 square feet or less, may increase the maximum impervious coverage to 65 percent;

(d)	a lot that is aggregated with other property to form a site may not be disaggregated to satisfy this subsection; and

(6)	may apply the requirements of Section 25-2-1602 (Front Porch Setback), Section 25-2-1603 (Impervious Cover and Parking Placement Requirements), or Section 25-2-1604 (Garage Placement) to the district or a designated portion of the district;

(7)	may restrict front yard parking by including all or a portion of the district in the restricted parking area map described in Section 12-5-29 (Front or Side Yard Parking);

(8)	may apply the requirements of Section 25-2-812(N) (Mobile Food Establishments) to the district or a designated portion of the district;

(9)	may modify the following requirements of Subchapter F (Residential Design And Compatibility Standards) for the district or a designated portion of the district:

(a)	the maximum floor-to-area ratio and maximum square footage of gross floor area prescribed by Subchapter F (Residential Design And Compatibility Standards);

(b)	the maximum linear feet of gables or dormers protruding from the setback plane;

(c)	the height of the side and rear setback planes;

(d)	the minimum front yard setback requirement; and

(10)	may apply the requirements of Section 25-2-1407 (Affordable Housing) to the district or a designated portion of the district.

Source: Ord. 000406-81; Ord. 020718-83; 030424-57; Ord. 030925-64; Ord. 031211-11; Ord. 040325-Z-1; Ord. 20060216-043; Ord. 20060309-058; Ord. 20060622-022; Ord. 20060928-020; Ord. 20060928-022; Ord. 20080618-093; Ord. 20080807-091; Ord. No. 20160303-036, Pt. 2, 3-14-16; Ord. No. 20240516-006, Pt. 7, 5-27-24.

§ 25-2-1407 AFFORDABLE HOUSING.

(A)	This section only applies to property that is subject to a NP combining district zoning or rezoning ordinance that applies the requirements of this section under Section 25-2-1406 (Ordinance Requirements).

(B)	A provision in this section applies only if:

(1)	the director of the Neighborhood Housing and Community Development Department certifies that the development complies with the City's S.M.A.R.T. Housing Program; and

(2)	ten percent or more of the dwelling units are reserved for a period of not less than 20 years for rental or purchase by an occupant whose gross household income does not exceed 60 percent of the median family income for the Austin metropolitan statistical area.

(C)	This subsection applies in a single family residence standard lot (SF-2) district or single family residence (SF-3) district.

(1)	The maximum impervious cover is 50 percent if the director of the Watershed Protection and Development Review Department determines that the development will not result in additional identifiable adverse flooding on other property.

(2)	A noncomplying structure may be replaced with a new structure if the new structure does not increase the existing degree of noncompliance with yard setbacks.

(D)	This subsection applies to a duplex residential use.

(1)	The minimum lot area is 5,750 square feet.

(2)	The maximum impervious cover is 50 percent if the director of the Watershed Protection and Development Review Department determines that the development will not result in additional identifiable adverse flooding on other property.

(3)	A maximum of eight bedrooms are permitted.

(E)	This subsection applies to a two family residential use.

(1)	The minimum lot area is 5,750 square feet.

(2)	The maximum impervious cover is 50 percent if the director of the Watershed Protection and Development Review Department determines that the development will not result in additional identifiable adverse flooding on other property.

(3)	The second dwelling unit may not exceed a gross floor area of 850 square feet. All of the allowed gross floor area may be on the second story, if any. The gross floor area limitation does not apply to a lot with 7,000 or more square feet of area.

(F)	This subsection applies to a secondary apartment special use.

(1)	The maximum impervious cover is 50 percent if the director of the Watershed Protection and Development Review Department determines that the development will not result in additional identifiable adverse flooding on other property.

(2)	The second dwelling unit may not exceed a gross floor area of 850 square feet. All of the allowed gross floor area may be on the second story, if any. The gross floor area limitation does not apply to a lot with 7,000 or more square feet of area.

Source: Ord. 20080807-091.

#### ARTICLE 2. URBAN HOME SPECIAL USE.

§ 25-2-1421 APPLICABILITY OF ARTICLE.

This article applies to an urban home special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1422 URBAN HOME PERMITTED IN CERTAIN ZONING DISTRICTS.

An urban home special use is permitted in the following zoning districts:

(1)	family residence (SF-3) district;

(2)	urban family residence (SF-5) district;

(3)	townhouse and condominium residence (SF-6) district;

(4)	multifamily residence limited density (MF-1) district;

(5)	multifamily residence low density (MF-2) district;

(6)	multifamily residence medium density (MF-3) district;

(7)	multifamily residence moderate-high density (MF-4) district;

(8)	multifamily residence high density (MF-5) district;

(9)	multifamily residence highest density (MF-6) district; and

(10)	mixed use (MU) combining district.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 041118-57.

§ 25-2-1423 COMMUNITY OPEN SPACE.

For an urban home special use development of more than eight lots, 250 square feet of community open space for each lot is required.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1424 URBAN HOME REGULATIONS.

(A)	For an urban home special use:

(1)	the minimum lot size is 3,500 square feet;

(2)	the minimum lot width is 35 feet;

(3)	the maximum height of a structure is 35 feet;

(4)	the minimum street side yard setback is 10 feet;

(5)	the minimum interior side yard setback is 5 feet;

(6)	the minimum rear yard setback is 5 feet;

(7)	the maximum building coverage is 55 percent; and

(8)	the maximum impervious coverage is:

(a)	the percentage prescribed by the zoning base district site development regulations; or

(b)	for a lot with an area of not more than 4,000 square feet, 65 percent.

(B)	Except as otherwise provided in this subsection, the minimum front yard setback is 20 feet.

(1)	If urban home special uses are proposed for the entire length of a block face, the minimum front yard setback is 15 feet.

(2)	For an urban home special use that adjoins a legally developed lot with a front yard setback of less than 25 feet, the minimum front yard setback is equal to the average of the front yard setbacks applicable to adjoining lots.

(C)	For an urban home special use with a front driveway:

(1)	the garage, if any, must be at least five feet behind the front façade of the principal structure; and

(2)	for a garage within 20 feet of the front façade, the width of the garage may not exceed 50 percent of the width of the front façade.

(D)	The maximum driveway width in a front yard or street side yard is:

(1)	12 feet; or

(2)	if the driveway serves two or more dwelling units;

(a)	18 feet for a one-way driveway; or

(b)	24 feet for a two-way driveway.

(E)	Other than in a driveway, parking is not permitted in a front yard of an urban home special use.

(F)	The main entrance of an urban home special use must face the front lot line, except on a flag lot.

(G)	A covered front porch at entry level is required for an urban home special use, except on a flag lot.

(1)	The minimum depth of the porch is five feet.

(2)	The minimum width of the porch is 50 percent of the width of the building façade.

(H)	Two hundred square feet of private open space is required for each dwelling.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

#### ARTICLE 3. COTTAGE SPECIAL USE.

§ 25-2-1441 APPLICABILITY OF ARTICLE.

This article applies to a cottage special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1442 COTTAGE PERMITTED IN CERTAIN ZONING DISTRICTS.

A cottage special use is permitted in the following zoning districts:

(1)	family residence (SF-3) district;

(2)	urban family residence (SF-5) district;

(3)	townhouse and condominium residence site (SF-6) district;

(4)	multifamily residence limited density (MF-1) district;

(5)	multifamily residence low density (MF-2) district;

(6)	multifamily residence medium density (MF-3) district;

(7)	multifamily residence moderate-high density (MF-4) district;

(8)	multifamily residence high density (MF-5) district;

(9)	multifamily residence highest density (MF-6) district; and

(10)	mixed use (MU) combining district.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 041118-57.

§ 25-2-1443 DEVELOPMENT REQUIREMENTS.

(A)	A cottage special use development may not exceed two acres in size.

(B)	For a cottage special use development of more than eight lots, 250 square feet of community open space is required for each lot. The community open space requirement is in addition to the parkland dedication requirement of Chapter 25-4, Article 3, Division 5 (Parkland Dedication).

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

§ 25-2-1444 COTTAGE REGULATIONS.

(A)	For a cottage special use:

(1)	the minimum lot width is 30 feet;

(2)	the maximum height of a structure is 35 feet;

(3)	the minimum front yard setback is 15 feet;

(4)	the minimum street side yard setback is 10 feet;

(5)	the minimum interior side yard setback is 5 feet;

(6)	the minimum rear yard setback is 5 feet;

(7)	the maximum building coverage is 55 percent; and

(8)	the maximum impervious coverage is:

(a)	the percentage prescribed by the base zoning district site development regulations; or

(b)	for a lot with an area of not more than 4,000 square feet, 65 percent.

(B)	The minimum lot area for a cottage special use is:

(1)	2,500 square feet; or

(2)	3,500 square feet for a lot that is located in a SF-3 district; and

(a)	is a corner lot; or

(b)	adjoins a lot that is:

(i)	zoned SF-3;

(ii)	has a lot area of at least 5,750 square feet; and

(iii)	is developed as a single-family residence.

(C)	For a cottage special use with a front driveway, a garage, if any, must be at least five feet behind the front façade of the principal structure.

(D)	The maximum driveway width in a front yard or street side yard is:

(1)	12 feet; or

(2)	if the driveway serves two or more dwelling units:

(a)	18 feet for a one-way driveway; or

(b)	24 feet for a two-way driveway.

(E)	Other than in a driveway, parking is not permitted in a front yard of a cottage special use.

(F)	The main entrance of the principal structure must face the front lot line, except on a flag lot.

(G)	A covered front porch at entry level is required for a cottage special use, except on a flag lot.

(1)	The minimum depth of the porch is five feet.

(2)	The minimum width of the porch is 50 percent of the width of the front façade.

(H)	Two hundred square feet of private open space is required for each dwelling.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

#### ARTICLE 4. SECONDARY APARTMENT SPECIAL USE.

§ 25-2-1461 APPLICABILITY OF ARTICLE.

This article applies to a secondary apartment special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1462 SECONDARY APARTMENT PERMITTED IN CERTAIN ZONING DISTRICTS.

A secondary apartment special use is permitted in the following zoning districts:

(1)	single-family residence large lot (SF-1) district;

(2)	single-family residence standard lot (SF-2) district;

(3)	family residence (SF-3) district;

(4)	urban family residence (SF-5) district;

(5)	townhouse and condominium residence (SF-6) district;

(6)	multifamily residence limited density (MF-1) district;

(7)	multifamily residence low density (MF-2) district;

(8)	multifamily residence medium density (MF-3) district;

(9)	multifamily residence moderate-high density (MF-4) district;

(10)	multifamily residence high density (MF-5) district;

(11)	multifamily residence highest density (MF-6) district; and

(12)	mixed use (MU) combining district.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 041118-57.

§ 25-2-1463 SECONDARY APARTMENT REGULATIONS.

(A)	A secondary apartment is not permitted in combination with a cottage or urban home special use.

(B)	A secondary apartment must be located in a structure other than the principal structure.

(C)	The secondary apartment:

(1)	must be contained in a structure other than the principal structure;

(2)	must be located:

(a)	at least 10 feet to the rear or side of the principal structure; or

(b)	above a detached garage;

(3)	may be connected to the principal structure by a covered walkway;

(4)	may not exceed a height of 30 feet, and is limited to two stories;

(5)	may not exceed:

(a)	1,100 total square feet or a floor-to-area ratio of 0.15, whichever is smaller; and

(b)	550 square feet on the second story, if any.

(D)	Impervious cover for the site may not exceed 45 percent.

(E)	Building cover for the site may not exceed 40 percent.

Source: Ord. 000406-81; Ord. 031120-44; Ord. 031211-11; Ord. 041118-59; Ord. No. 20151119-080, Pt. 2, 11-30-15; Ord. No. 20250227-039, Pt. 3, 10-1-25.

#### ARTICLE 5. CORNER STORE SPECIAL USE.

§ 25-2-1481 APPLICABILITY OF ARTICLE.

This article applies to a corner store special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1482 CORNER STORE PERMITTED IN CERTAIN ZONING DISTRICTS.

A corner store special use is permitted in the following base districts:

(1)	family residence (SF-3) district;

(2)	urban family residence (SF-5) district;

(3)	townhouse and condominium residence (SF-6) district;

(4)	multifamily residence limited density (MF-1) district;

(5)	multifamily residence low density (MF-2) district;

(6)	multifamily residence medium density (MF-3) district;

(7)	multifamily residence moderate-high density (MF-4) district;

(8)	multifamily residence high density (MF-5) district; and

(9)	multifamily residence highest density (MF-6) district.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1483 CORNER STORE PERMITTED USES.

(A)	A corner store special use is limited to the following commercial uses:

(1)	art gallery;

(2)	consumer convenience services;

(3)	consumer repair services;

(4)	food sales;

(5)	general retail sales (convenience);

(6)	personal services;

(7)	restaurant (general); and

(8)	restaurant (limited).

(B)	A maximum of one dwelling unit is permitted in a corner store structure.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 040617-Z-1.

§ 25-2-1484 CORNER STORE LOCATION.

(A)	A corner store must be located at a street intersection.

(B)	A corner store may not be located within 600 feet of another corner store.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1485 CORNER STORE REGULATIONS.

(A)	For a corner store special use:

(1)	the minimum lot area is 5,750 square feet;

(2)	the minimum lot width is 50 feet;

(3)	the maximum building height is 35 feet;

(4)	the minimum front yard setback is 5 feet;

(5)	the maximum front yard setback is 15 feet;

(6)	the minimum street side yard setback is 10 feet;

(7)	the minimum interior side yard setback is 5 feet;

(8)	the minimum rear yard setback is 10 feet;

(9)	the maximum building coverage is the lesser of 55 percent or 3,000 square feet; and

(10)	the maximum impervious coverage is 65 percent.

(B)	A corner store may not include a drive through facility.

(C)	A corner store may not be open to the public between the hours of 11:00 p.m. and 6:00 a.m.

(D)	Exterior lighting:

(1)	must be hooded or shielded so that the light source is not directly visible across the source property line; and

(2)	may not exceed 0.4 foot candles across the source property line.

(E)	A building façade:

(1)	may not extend horizontally in an unbroken line for more than 30 feet;

(2)	must include windows, balconies, porches, stoops, or similar architectural features;

(3)	must have awnings along at least 50 percent of the length of the ground floor façade; and

(4)	at least 50 percent of the wall area of the ground floor façade must consist of doors or clear or lightly tinted windows.

(F)	A street yard of 1,000 square feet or less is not required to be landscaped, and a parking area with 12 or fewer parking spaces is not required to have landscaped islands, peninsulas, or medians.

(G)	The outdoor seating area, if any, for a restaurant (limited) use may not exceed 50 percent of the indoor seating area.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11; Ord. 031211-41.

#### ARTICLE 6. NEIGHBORHOOD MIXED USE BUILDING SPECIAL USE.

§ 25-2-1501 APPLICABILITY OF ARTICLE.

This article applies to a neighborhood mixed use building special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1502 NEIGHBORHOOD MIXED USE BUILDING PERMITTED IN CERTAIN ZONING DISTRICTS.

A neighborhood mixed use building special use is permitted in the following zoning base districts:

(1)	limited office (LO) district;

(2)	general office (GO) district;

(3)	neighborhood commercial (LR) district;

(4)	community commercial (GR) district;

(5)	general commercial services (CS) district;

(6)	commercial-liquor sales (CS-1) district;

(7)	commercial highway services (CH) district; and

(8)	limited industrial services (LI) district.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1503 RESIDENTIAL USES IN NEIGHBORHOOD MIXED USE BUILDING.

(A)	A neighborhood mixed use building special use must contain a residential use.

(B)	A residential use in a neighborhood mixed use building special use may be located:

(1)	above the ground floor; and

(2)	in not more than 50 percent of the gross floor area of the ground floor.

(C)	For a multi-story building, not less than 50 percent of the gross floor area above the ground floor must be used for a residential use.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 20080424-029.

§ 25-2-1504 NEIGHBORHOOD MIXED USE BUILDING REGULATIONS.

(A)	For a neighborhood mixed use building special use:

(1)	the maximum site area is one acre;

(2)	the minimum lot size is 5,750 square feet;

(3)	the minimum lot width is 50 feet;

(4)	the minimum street side yard setback is 10 feet;

(5)	the minimum front yard setback is:

(a)	5 feet; or

(b)	for a LO or LR district, 10 feet; and

(6)	the maximum front yard setback is:

(a)	10 feet; or

(b)	for a LO or LR district, 15 feet.

(B)	For a neighborhood mixed use building special use adjacent to a roadway with not more than two lanes, the building height may not exceed 40 feet.

(C)	The building façade of a neighborhood mixed use building:

(1)	may not extend horizontally in an unbroken line for more than 30 feet;

(2)	must include windows, balconies, porches, stoops, or similar architectural features;

(3)	must have awnings along at least 50 percent of the length of the ground floor façade; and

(4)	at least 50 percent of the wall area of the ground floor façade must consist of doors or windows with a visible light transmittance rating of 0.6 or higher.

(D)	This subsection prescribes parking requirements:

(1)	Parking in front of a neighborhood mixed use building, other than on a street, is prohibited.

(2)	At least 50 percent of any parking that is provided must be located to the rear of the building.

(E)	Exterior lighting:

(1)	must be shielded so that the light source is not directly visible across the source property line; and

(2)	may not exceed 0.4 foot candles across the source property line.

(F)	A street yard of 1,000 square feet or less is not required to be landscaped, and a parking area with 12 or fewer parking spaces is not required to have landscaped islands, peninsulas, or medians.

(G)	A neighborhood mixed use building may not include a drive through facility.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11; Ord. No. 20231102-028, Pt. 24, 11-13-23.

#### ARTICLE 7. RESIDENTIAL INFILL AND NEIGHBORHOOD URBAN CENTER SPECIAL USES.

##### Division 1. Development Plan.

§ 25-2-1521 DEVELOPMENT PLAN REQUIRED.

(A)	A person may not use or develop property as a residential infill or neighborhood urban center special use unless the Planning Commission approves a development plan under this division.

(B)	A development plan must include:

(1)	the locations of land uses, number of dwelling units, and approximate gross floor area of each use;

(2)	the layout of the transportation network;

(3)	the location, size, and type of each community open space area;

(4)	the location and type of each drainage or water quality control facility;

(5)	the location of the 100-year flood plain;

(6)	the location of each critical environmental feature; and

(7)	additional information required by the director of the Neighborhood Planning and Zoning Department to demonstrate compliance with this subchapter.

Source: Ord. 000406-81; Ord. 010329-18; Ord. 031211-11.

§ 25-2-1522 SUBMITTAL AND APPROVAL OF DEVELOPMENT PLAN.

(A)	An applicant must submit the development plan to the director of the Neighborhood Planning and Zoning Department.

(B)	The director of the Neighborhood Planning and Zoning Department shall review the development plan and make a recommendation to the Planning Commission.

(C)	The Planning Commission shall approve the development plan after a determination that the plan meets the requirements of Section 25-2-1523 (Development Plan Approval Criteria).

(D)	If the Planning Commission denies the development plan, the commission shall identify the basis of the denial.

Source: Ord. 000406-81; Ord. 010329-18; Ord. 031211-11.

§ 25-2-1523 DEVELOPMENT PLAN APPROVAL CRITERIA.

A development plan must:

(1)	demonstrate compliance with the requirements of Division 2 (Residential Infill Special Use) or Division 3 (Neighborhood urban center Special Use), as applicable;

(2)	be designed to promote pedestrian activity and the use of mass transit;

(3)	propose building height, bulk, and scale that is compatible with adjacent single-family development, if any; and

(4)	include high quality community open space as an organizing feature.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1524 DEVELOPMENT PLAN REVISIONS.

(A)	Except as provided in Subsection (B), the approval of the Planning Commission is required to revise a development plan. The revision must comply with Section 25-2-1523 (Development Plan Approval Criteria).

(B)	The director of the Neighborhood Planning and Zoning Department may approve a minor revision to a development plan if the director of the Neighborhood Planning and Zoning Department determines that the revised plan complies with the applicable requirements of this subchapter. The following are minor revisions:

(1)	a change in the location of a land use, if the director of the Neighborhood Planning and Zoning Department determines that the basic layout of the development plan remains the same, and that the proposed change does not negatively affect existing adjacent land uses;

(2)	a reduction in the number of dwelling units;

(3)	a reduction in the total gross floor area of the commercial uses;

(4)	a change in the mix of residential uses;

(5)	a change in the transportation network if the director of the Neighborhood Planning and Zoning Department determines that the basic layout of the development plan remains the same;

(6)	a change in the size or location of community open space, if the director of the Neighborhood Planning and Zoning Department determines that the quality and functionality of the overall community open space is not reduced;

(7)	a change in the location or type of a drainage or water quality control facility, if the director of the Neighborhood Planning and Zoning Department determines that the basic layout of the development plan remains the same;

(8)	a change in the location or type of an critical environmental feature, if the director of the Neighborhood Planning and Zoning Department determines that the revision more accurately describes the feature; and

(9)	a change in the location of a 100-year floodplain, if the director of the Neighborhood Planning and Zoning Department determines that the revision more accurately describes the floodplain.

(C)	An interested party may appeal the director of the Neighborhood Planning and Zoning Department's decision under Subsection (B) to the Planning Commission.

Source: Ord. 000406-81; Ord. 010329-18; Ord. 031211-11.

##### Division 2. Residential Infill Special Use.

§ 25-2-1531 APPLICABILITY OF DIVISION.

This division applies to a residential infill special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1532 RESIDENTIAL INFILL PERMITTED IN CERTAIN ZONING DISTRICTS.

A residential infill special use is permitted in the following zoning base districts:

(1)	family residence (SF-3) district;

(2)	urban family residence (SF-5) district;

(3)	townhouse and condominium residence (SF-6) district;

(4)	multifamily residence limited density (MF-1) district;

(5)	multifamily residence low density (MF-2) district;

(6)	multifamily residence medium density (MF-3) district;

(7)	multifamily residence moderate-high density (MF-4) district;

(8)	multifamily residence high density (MF-5) district;

(9)	multifamily residence highest density (MF-6) district; and

(10)	limited industrial services (LI) district.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1533 RESIDENTIAL INFILL PERMITTED USES.

(A)	A residential infill special use is limited to the land uses prescribed by this section.

(B)	The following residential uses are permitted:

(1)	single family residential;

(2)	duplex residential;

(3)	townhouse residential;

(4)	condominium residential;

(5)	multifamily residential;

(6)	urban home special use, in accordance with Article 2 (Urban Home Special Use);

(7)	cottage special use, in accordance with Article 3 (Cottage Special Use); and

(8)	secondary apartment special use, in accordance with Article 4 (Secondary Apartment Special Use).

(C)	The following commercial uses are permitted:

(1)	art gallery;

(2)	consumer convenience services;

(3)	consumer repair services;

(4)	food sales;

(5)	general retail sales (convenience);

(6)	personal services;

(7)	restaurant (general) without drive-in service; and

(8)	restaurant (limited) without drive-in service.

(D)	The civic uses described in Section 25-2-6 (Civic Uses Described) are permitted in accordance with the requirements of the zoning base district.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 031211-41; Ord. 040617-Z-1.

§ 25-2-1534 DEVELOPMENT REQUIREMENTS.

(A)	A residential infill special use development must have a site area of:

(1)	at least one acre; and

(2)	not more than 40 acres.

(B)	This subsection prescribes land use allocation requirements for a residential infill special use development.

(1)	Commercial uses are limited to 1,000 square feet of gross floor area for each full acre included in the residential infill special use development.

(2)	At least 40 percent, and not more than 80 percent, of the dwelling units must be a single-family residential use, cottage special use, or urban home special use.

(3)	Not more than 20 percent of the dwelling units may be cottage special uses.

(4)	Not more than 10 percent of the dwelling units may be duplex residential uses.

(5)	At least 10 percent of the dwelling units must be townhouse or multifamily uses.

(6)	Not more than 20 percent of the dwelling units may be a multifamily use other than a condominium use.

(7)	For a development of not more than five acres, at least 10 percent of the development's area must be community open space.

(8)	For a development of more than five acres, at least 20 percent of the development's area must be community open space.

(C)	Single-family residential uses are required for the portion of the development that adjoins land that is:

(1)	zoned SF-3 or more restrictive; or

(2)	used for a use permitted in a SF-3 or more restrictive district.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

§ 25-2-1535 DUPLEX REGULATIONS.

For a duplex residential use:

(1)	the minimum lot area is 5,750 square feet;

(2)	the minimum lot width is 50 feet;

(3)	the minimum front setback is 15 feet;

(4)	the minimum street side setback is 10 feet;

(5)	the minimum interior side yard setback is five feet;

(6)	the minimum rear yard setback is five feet;

(7)	the maximum height is 35 feet;

(8)	the maximum building coverage is 50 percent; and

(9)	the maximum impervious coverage is 55 percent.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1536 TOWNHOUSE REGULATIONS.

(A)	For a townhouse residential use:

(1)	the minimum lot area is 2,000 square feet;

(2)	the minimum lot width is 20 feet;

(3)	the minimum front setback is five feet;

(4)	the maximum front setback is 10 feet;

(5)	the minimum street side setback is 10 feet;

(6)	the minimum interior side yard setback is zero feet;

(7)	the minimum rear yard setback is five feet;

(8)	the maximum building height is 35 feet;

(9)	the maximum building coverage is 55 percent; and

(10)	the maximum impervious coverage is 65 percent.

(B)	The finished floor elevation of the first floor of a townhouse must be at least 18 inches above the elevation of the sidewalk at the front lot line.

(C)	A townhouse group is limited to 10 townhouses.

(D)	Vehicular access to a townhouse group must be:

(1)	through a public alley or dedicated access easement at the rear of the group; or

(2)	through a single front driveway that provides access to the rear of the group.

(E)	Other than in a garage, parking is permitted only at the rear of a townhouse. A parking area must be screened from the street.

(F)	A lot may contain not more than one townhouse.

(G)	Two hundred square feet of private open space is required for each townhouse.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1537 MULTIFAMILY AND CONDOMINIUM REGULATIONS.

(A)	For a multifamily residential use, including a condominium residential use:

(1)	the minimum lot area is 5,750 square feet;

(2)	the minimum lot width is 50 feet;

(3)	the maximum front setback is 10 feet;

(4)	the minimum front setback is five feet;

(5)	the minimum street side yard setback is 10 feet;

(6)	the minimum interior side yard setback is five feet;

(7)	the minimum rear yard setback is 10 feet;

(8)	the maximum building height is 35 feet;

(9)	the maximum building coverage is 55 percent;

(10)	the maximum impervious coverage is 65 percent;

(11)	the maximum building footprint is 4,000 square feet; and

(12)	a building may contain not more than 12 dwelling units.

(B)	One hundred square feet of private open space is required for each multifamily dwelling.

(C)	Two hundred square feet of private open space is required for each condominium dwelling.

(D)	Parking is not permitted in a front yard.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1538 COMMERCIAL REGULATIONS.

(A)	For a commercial use:

(1)	the minimum lot area is 2,500 square feet;

(2)	the minimum lot width is 30 feet;

(3)	the maximum front yard setback is 10 feet;

(4)	the minimum front yard setback is five feet;

(5)	the minimum street side yard setback is 10 feet;

(6)	the minimum interior yard setback is five feet;

(7)	the minimum rear yard setback is 10 feet;

(8)	the maximum building height is 35 feet;

(9)	the maximum building coverage is 55 percent;

(10)	the maximum impervious cover is 65 percent; and

(11)	the maximum building footprint is 5,000 square feet.

(B)	A commercial use may not be open to the public between the hours of 11:00 p.m. and 6:00 a.m.

(C)	The outdoor seating area, if any, for a restaurant (limited) use may not exceed 50 percent of the indoor seating area.

Source: Ord. 000406-81; Ord. 031211-11; Ord. 031211-41.

§ 25-2-1539 COMMUNITY OPEN SPACE.

Not more than 50 percent of the community open space may be plazas or squares.

Source: Ord. 000406-81; Ord. 031211-11.

##### Division 3. Neighborhood Urban Center Special Use.

§ 25-2-1551 APPLICABILITY OF DIVISION.

This division applies to a neighborhood urban center special use.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1552 NEIGHBORHOOD URBAN CENTER SPECIAL USE PERMITTED IN CERTAIN ZONING DISTRICTS.

A neighborhood urban center special use is permitted in the following zoning base districts:

(1)	limited office (LO) district;

(2)	general office (GO) district;

(3)	neighborhood commercial (LR) district;

(4)	community commercial (GR) district;

(5)	general commercial services (CS) district;

(6)	commercial-liquor sales (CS-1) district; and

(7)	limited industrial services (LI) district.

Source: Ord. 000406-81; 030424-57; Ord. 031211-11.

§ 25-2-1553 NEIGHBORHOOD URBAN CENTER PERMITTED USES.

(A)	A neighborhood urban center special use is limited to the land uses prescribed by this section.

(B)	The following residential uses are permitted:

(1)	townhouse residential;

(2)	condominium residential; and

(3)	multifamily residential.

(C)	A commercial use is permitted, conditional, or prohibited as prescribed by the base zoning district regulations, except that a use with a drive-in service is prohibited and a service station use allowed as a permitted use is a conditional use.

(D)	The civic uses described in Section 25-2-6 (Civic Uses Described) are permitted in accordance with the requirements of the zoning base district.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

§ 25-2-1554 DEVELOPMENT REQUIREMENTS.

(A)	A neighborhood urban center special use development must have a site area of:

(1)	at least one acre; and

(2)	not more than 40 acres.

(B)	This subsection prescribes land use allocation requirements for a neighborhood urban center special use development.

(1)	At least 10 percent of the development's gross floor area must be used for commercial uses.

(2)	At least 25 percent of the development's gross floor area must be used for residential uses.

(3)	At least 20 percent of the development's dwelling units must be townhouses or condominiums.

(4)	For a project of not more than 5 acres, at least 10 percent of the development's area must be community open space.

(5)	For a development of more than 5 acres, at least 20 percent of the development's area must be community open space.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

§ 25-2-1555 TOWNHOUSE REGULATIONS.

(A)	For a townhouse residential use:

(1)	the minimum lot area is 2,000 square feet;

(2)	the minimum lot width is 20 feet;

(3)	the minimum front setback is five feet;

(4)	the maximum front setback is 10 feet;

(5)	the minimum street side setback is 10 feet;

(6)	the minimum interior side yard setback is zero feet;

(7)	the minimum rear yard setback is five feet;

(8)	the maximum building height is 35 feet;

(9)	the maximum building coverage is 55 percent; and

(10)	the maximum impervious coverage is 65 percent.

(B)	The finished floor elevation of the first floor of a townhouse must be at least 18 inches above the elevation of the sidewalk at the front lot line.

(C)	Vehicular access to a townhouse group must be:

(1)	through a public alley or dedicated access easement at the rear of the group; or

(2)	through a single front driveway that provides access to the rear of the group.

(D)	Other than in a garage, parking is permitted only at the rear of a townhouse. A parking area must be screened from the street.

(E)	A lot may contain not more than one townhouse.

(F)	Two hundred square feet of private open space is required for each townhouse.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1556 MULTIFAMILY AND CONDOMINIUM REGULATIONS.

(A)	For a multifamily residential use, including a condominium use:

(1)	the minimum lot size is 3,500 square feet;

(2)	the minimum lot width is 50 feet;

(3)	the maximum height is 60 feet;

(4)	the maximum front setback is 10 feet;

(5)	the minimum front setback is five feet;

(6)	the minimum street side yard setback is 10 feet;

(7)	the minimum interior side yard setback is five feet;

(8)	the minimum rear yard setback is 10 feet;

(9)	the maximum building coverage is 70 percent;

(10)	the maximum impervious coverage is 80 percent; and

(11)	the maximum building footprint is 5,000 square feet.

(B)	One hundred square feet of private open space is required for each multifamily dwelling.

(C)	Two hundred square feet of private open space is required for each condominium dwelling.

(D)	Parking is not permitted in a front yard.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11; Ord. No. 20231102-028, Pt. 25, 11-13-23.

§ 25-2-1557 COMMERCIAL USE REGULATIONS.

For a commercial use:

(1)	the minimum lot size is 3,500 square feet;

(2)	the minimum lot width is 50 feet;

(3)	the maximum height is 60 feet;

(4)	the maximum front yard setback is 10 feet;

(5)	the minimum front yard setback is five feet; and

(6)	the minimum street side yard setback is 10 feet.

Source: Ord. 000406-81; Ord. 030424-57; Ord. 031211-11.

##### Division 4. Additional Development Requirements.

§ 25-2-1561 APPLICABILITY OF DIVISION.

This division prescribes additional development requirements for residential infill and neighborhood urban center special uses.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1562 COMMUNITY OPEN SPACE.

In addition to other community open space requirements prescribed by this subchapter:

(1)	each community open space area must be at least 500 square feet in size and at least 20 feet across in each direction;

(2)	the aggregate impervious cover for all community open spaces may not exceed 50 percent; and

(3)	a plaza or square may not exceed 90 percent impervious cover.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1563 DRIVE-THROUGH FACILITIES PROHIBITED.

Drive-through facilities and other facilities that allow people to remain in vehicles while receiving products or services are prohibited. This prohibition does not apply to the fueling facilities of a service station.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1564 COMPATIBILITY STANDARDS.

(A)	The compatibility standards of Chapter 25-2, Article 10 (Compatibility Standards) apply only to property along the perimeter of an area used or developed as a residential infill or neighborhood urban center special use.

(B)	Within an area used or developed as a residential infill or neighborhood urban center special use, the compatibility standards of Section 25-3-86 (Compatibility Standards) apply.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1565 ROADWAY DESIGN.

(A)	A new road within a residential infill or neighborhood urban center special use project must comply with the roadway design standards of the Traditional Neighborhood District Criteria Manual.

(B)	The director of the Neighborhood Planning and Zoning Department may approve the use of an innovative roadway design that is not described in the Traditional Neighborhood District Criteria Manual.

Source: Ord. 000406-81; Ord. 010329-18; Ord. 031211-11.

§ 25-2-1566 RESERVED.

Ord. No. 20231102-028, Pt. 26, effective November 13, 2023, repealed § 25-2-1566, which pertained to commercial use parking requirements and derived from Ord. 000406-81; Ord. 031211-11.

§ 25-2-1567 RESIDENTIAL USES IN COMMERCIAL BUILDINGS.

A residential use may be located above the ground floor of a commercial building.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1568 SPECIAL FEATURES FOR COMMERCIAL AND MULTIFAMILY BUILDINGS.

The building façade of a commercial building or a multifamily residential building:

(1)	may not extend horizontally in an unbroken line for more than 30 feet;

(2)	must include windows, balconies, porches, stoops, or similar architectural features;

(3)	must have awnings along at least 50 percent of the length of the ground floor building façade; and

(4)	at least 50 percent of the wall area of the ground floor building façade must consist of doors or clear or lightly tinted windows.

Source: Ord. 000406-81; Ord. 031211-11.

§ 25-2-1569 LANDSCAPING.

A street yard of 1,000 square feet or less is not required to be landscaped, and a parking area with 12 or fewer parking spaces is not required to have landscaped islands, peninsulas, or medians.

Source: Ord. 000406-81; Ord. 031211-11.

#### ARTICLE 8. ADDITIONAL REQUIREMENTS FOR CERTAIN DISTRICTS.

§ 25-2-1601 APPLICABILITY.

A section in this article applies to a NP combining district or a designated portion of a district only if the ordinance establishing the district provides for applicability in accordance with Section 25-2-1406 (Ordinance Requirements).

Source: Ord. 030925-64; Ord. 031211-11.

§ 25-2-1602 FRONT PORCH SETBACK.

(A)	This section applies to a single-family residential use, a duplex residential use, or a two-family residential use.

(B)	In this section, "porch" means a porch that is open on three sides and that may have a roof.

(C)	Subject to the limitations of this section, a porch may project into a front yard.

(D)	Except as provided in Subsection (E), the porch must be at least 15 feet from the front lot line.

(E)	A porch roof overhang or porch step must be at least 13 feet from the front lot line.

Source: Ord. 030925-64; Ord. 031211-11

§ 25-2-1603 IMPERVIOUS COVER AND PARKING PLACEMENT RESTRICTIONS.

(A)	This section applies to a single-family residential use, a duplex residential use, or a two-family residential use.

(B)	Except as provided in Subsection (C), impervious cover in a front yard may not exceed 40 percent.

(C)	The director may waive Subsection (B) if the director determines backing a motor vehicle onto the adjacent roadway is unsafe and that a circular driveway or turnaround in the front yard is required.

(D)	Not more than four parking spaces may be located in the front street yard, or for a corner lot, not more than four parking spaces may be located in the front street yard and side street yard combined.

Source: Ord. 030925-64; Ord. 031211-11.

§ 25-2-1604 GARAGE PLACEMENT.

(A)	This section applies to a single-family residential use, a duplex residential use, or a two-family residential use.

(B)	In this section:

(1)	BUILDING FAÇADE means the front-facing exterior wall or walls of the first floor of the principal structure on a lot, and the term excludes the building façade of the portion of the principal structure designed or used as a parking structure. Projections from front-facing exterior walls, including but not limited to eaves, chimneys, porches, stoops, box or bay windows, and other similar features as determined by the building official, are not considered part of the building façade.

(2)	PARKING STRUCTURE means a garage or carport, either attached or detached from the principal structure.

(C)	A parking structure with an entrance that faces the front yard abutting public right-of-way:

(1)	may not be closer to the front lot line than the front-most exterior wall of the first floor of the building façade; and

(2)	if the parking structure is less than 20 feet behind the building façade, the width of the parking structure may not exceed 50 percent of the width of the building façade, measured parallel to the front lot line, or the line determined by the building official if located on an irregular lot.

Source: Ord. 030925-64; Ord. 031211-11; Ord. No. 20160505-051, Pt. 1, 5-16-16.

Editor's note(s)—Ord. No. 20240229-070, Pt. 1, effective March 11, 2024, repealed §§ 25-2-769.01—25-2-769.06, which pertained to Corridor Overlay and derived from Ord. No. 20221201-056, Pt. 3, 12-12-22; Ord. No. 20230608-088, Pts. 1, 2, 6-19-23.

Ord. No. 20250227-039, Pt. 1, effective October 1, 2025, repealed §§ 25-2-788—25-2-799, which pertained to requirements for short-term rental uses and derived from Ord. 20120802-122; Ord. 20130926-144; Ord. No. 20151119-080, Pts. 4—7, 11-30-15; Ord. No. 20151217-098, Pt 1, 12-28-15; Ord. No. 20160223-A.1, Pts. 1, 2, 3-5-16; Ord. No. 20231207-001, Pts. 10, 11, 12-18-23.

Ord. No. 20240516-004, Pt. 1, effective July 15, 2024, repealed the former Art. 10, §§ 25-2-1051, 25-2-1052, 25-2-1061—25-2-1068, 25-2-1081, 25-2-1082, and enacted a new Art. 10 as set out herein. The former Art. 10 pertained to similar subject matter and derived from Sections 13-2-25(2), 13-2-731, 13-2-733—13-2-739; Ord. 990225-70; Ord. 000309-39; Ord. 000406-81; Ord. 010329-18; Ord. 010607-8; Ord. 031211-11; Ord. 041202-16; Ord. 20050519-008; Ord. 20060216-043; Ord. 20060309-058; Ord. 20060504-039; Ord. 20060622-022; Ord. 20060928-022; Ord. 20061130-064; Ord. 20080618-093; Ord. 20090212-070; Ord. 20101209-075; Ord. 20131017-046; Ord. No. 20160922-080, Pt. 1, 10-3-16; Ord. No. 20231019-052, Pt. 9, 10-30-23; Ord. No. 20231102-028, Pt. 22, 11-13-23.